RDM Controller

User Guide

ESP32-S2 Thing Plus — Arduino Sketch

ROLITECH

# 1. Overview

This RDM Controller is firmware for an ESP32-S2 Thing Plus board that acts as an RDM (Remote Device Management) controller on a DMX512 bus. It discovers RDM-capable dimmer devices and provides an interactive serial command interface for controlling them. As well as the RDM features, there is also a basic DMX setup allowing any DMX channel to be set to a value between 0 and 255.

The command language deliberately mirrors the serial interface of the S3HW DMX dimmer (the RDM responder). This means an operator can use relevant identical commands whether they are connected directly to the dimmer over USB or remotely via the controller over RDM.

Primary use case: basic level settings, commissioning and diagnostic access to an S3HW Tempus retrofit dimmer rack without physical access to the dimmer unit.

# 2. Hardware

## 2.1 Controller Board

The controller runs on a SparkFun ESP32-S2 Thing Plus. The DMX interface is provided by a MAX485 transceiver connected as follows:

| Signal | ESP32-S2 Pin | Notes |
| --- | --- | --- |
| DMX TX | GPIO 34 | To MAX485 DI |
| DMX RX | GPIO 33 | From MAX485 RO |
| Direction (EN) | GPIO 3 | MAX485 DE+RE |
| DMX port | UART 1 | Port 0 reserved for USB serial |

## 2.2 Connection

Connect the controller to the same DMX bus as the S3HW dimmer. RDM requires a properly terminated RS-485 bus (120 Ω at each end). Connect USB to a computer running a serial terminal at 115200 baud.

# 3. Startup

On power-on the firmware:

- Initialises the DMX driver on UART 1

- Runs RDM discovery automatically

- Lists all found devices with their UIDs and manufacturer PID information

- Sets device [0] as the active device

- Prints Ready and waits for serial commands

Example startup  output:

=== S3HW RDM Controller ===

Discovering RDM devices...

1 device(s) found:

[0] 05e0:04298435  <-- active

Manufacturer PIDs:

0x8000: "Dimming Curve"  pdl=6  range=0-3  default=0

Ready. Type 'help' for commands.

Note: Discovery can take several seconds on a busy bus. This is normal behaviour — the RDM binary search tree requires multiple exchanges to resolve all UIDs.

# 4. Command Reference

Commands are entered via the serial monitor (115200 baud, newline terminated). Commands are case-sensitive and use the same names as the S3HW dimmer serial interface where applicable.

## 4.1 Discovery and Device Selection

| Command | Example | Description |
| --- | --- | --- |
| discover | discover | Re-run RDM discovery. Use after power-cycling a dimmer or connecting a new device. |
| dev <n> | dev 1 | Select active device by index (0-based). All subsequent commands operate on this device. |
| dev? | dev? | Show the index and UID of the currently active device. |

## 4.2 DMX Start Address

| Command | Example | Description |
| --- | --- | --- |
| addr <n> | addr 21 | SET the DMX start address (1–512) on the active device via RDM. Saved to NVS on the dimmer. |
| addr? | addr? | GET and display the current DMX start address from the active device. |

Note: The maximum valid address depends on the dimmer's current channel mode: 507 for 6×2.5kW mode, 510 for 3×5kW mode. The dimmer will NACK an out-of-range address.

## 4.3 Dimming Curves

Each of the 6 dimmer output channels has an independently selectable dimming curve. Curves are identified by a short name:

| Name | Full Name | Description |
| --- | --- | --- |
| sq | Square law | Perceptually linear brightness response. Default. |
| log | Logarithmic | Aggressive low-end response, best spread of perceived brightness. |
| lin | Linear | Firing angle proportional to DMX value. Reference curve. |
| tmp | Tempus analogue | Mathematical model of the original Rank Strand Tempus analogue fader response. Recommended for tungsten loads on Tempus hardware. |

| Command | Example | Description |
| --- | --- | --- |
| curve <name> | curve tmp | SET all 6 channels to the same curve via RDM. |
| curve <ch> <name> | curve 3 log | SET a single channel curve (ch 1–6). Other channels are not affected (0xFF mask). |
| curve? | curve? | GET and display the current curve for each channel. |

Note: When setting a single channel, the controller sends 0xFF in the array positions for all other channels. The dimmer interprets 0xFF as “keep existing value”, so untouched channels are preserved exactly.

## 4.4 Identify

| Command | Example | Description |
| --- | --- | --- |
| identify on | identify on | Activate RDM identify mode on the active device. The dimmer’s WS2812B status LED flashes white. |
| identify off | identify off | Deactivate identify mode. |
| identify? | identify? | GET current identify state (on or off). |

## 4.5 DMX Personality

DMX personalities define the operating mode and DMX footprint of a device. For the S3HW dimmer, personality 1 is the 6 x 2.5kW mode (footprint 6) and personality 2 is the 3 x 5kW mode (footprint 3). The controller can read the current personality and all available personalities with their labels, and can set the active personality via RDM.

| Command | Example | Description |
| --- | --- | --- |
| personality? | personality? | GET the current personality number and total count, then list each personality with its DMX footprint and label. The active personality is marked with <-- active. |
| personality <n> | personality 2 | SET the active personality to the 1-based index n. The dimmer saves the change to NVS. A reboot is recommended for the new footprint to take full effect on the RDM bus. |

Note: After setting a new personality, the dimmer resets the DMX start address to 1 if the current address would exceed the new footprint’s valid range. The footprint reported to other RDM controllers on the bus only updates after the dimmer reboots.

Example personality? output for an S3HW dimmer in 6-channel mode:

personality?

Personality: 1 of 2

[1] footprint=6   "6 x 2.5kW"  <-- active

[2] footprint=3   "3 x 5kW"

## 4.6 Basic Level Setting

## A constant DMX stream is output from the unit specifying a level for each of the 512 DMX channels. The level command allows the level of any channels to be set for testing purposes. On power up all levels are set to zero.

| Command | Example | Description |
| --- | --- | --- |
| level | level 1 255 | Set the DMX output level of channel 1 to 255 |

Note: The level command is write only, there is no level? command to display the current level of all 512 channels.

## 4.7 Information and Status

| Command | Example | Description |
| --- | --- | --- |
| info | Info | GET DEVICE_INFO (DMX footprint, sub-device count, sensor count) and list all manufacturer-specific PIDs with their descriptions. |
| status | Status | Combined summary: active device UID, DMX start address, and per-channel curve assignments. |
| help | Help | Print the command reference. |

## 4.8 Manufacturer PID Display

During discovery and when the info command is run, the controller queries SUPPORTED_PARAMETERS and then PARAMETER_DESCRIPTION for each PID in the manufacturer-specific range (0x8000–0xFFDF). The output for each PID shows:

- PID number — the 16-bit parameter ID in hex (e.g. 0x8000)

- Description string — human-readable name as registered by the device (e.g. “Dimming Curve”)

- pdl — parameter data length in bytes. For MFR_CURVE, pdl=6 means the payload is 6 bytes — one curve ID per output channel.

- range — minimum and maximum valid values. For MFR_CURVE, range=0-3 corresponds to the four curve IDs (square=0, log=1, linear=2, Tempus=3). The value 0xFF is also accepted as a “no change” mask on SET.

- default — factory default value. For MFR_CURVE, default=0 means square law on all channels.

Example output from the info command:

info

[0] 05e0:04298435

Footprint: 6  Sub-devices: 0  Sensors: 0

Manufacturer PIDs:

0x8000: "Dimming Curve"  pdl=6  range=0-3  default=0

If a device does not support PARAMETER_DESCRIPTION for a given PID, the output shows the NACK reason code instead. If no manufacturer PIDs are present, the line No manufacturer PIDs. is printed. Standard PIDs (below 0x8000) are not described here as they follow the E1.20 standard definitions.

# 5. Multi-Device Use

If discovery finds more than one RDM device on the bus, all devices are listed at startup. Use dev <n> to switch between them:

discover

2 device(s) found:

[0] 05e0:04298435  <-- active

[1] 05e0:0429F100

dev 1

Active device: [1] 05e0:0429F100

addr?

DMX start address: 7

| Command | Example | Description |
| --- | --- | --- |
| Dev | dev 1 | Set current (active) device to device 1 in the device list |

Note: All commands operate on the active device.

# 6. Worked Examples

## 6.1 Commissioning a new dimmer

discover                   ; find the dimmer

addr 1                     ; set DMX start address

curve tmp                  ; set Tempus curve on all channels

curve 6 sq                 ; override channel 6 to square law

status                     ; verify settings

## 6.2 Checking a dimmer is responding

identify on                ; flash the status LED

; visually confirm the correct dimmer is flashing

identify off

## 6.3 Changing address on a running rig

addr?                      ; check current address

addr 43                    ; set new address

addr?                      ; confirm

## 6.4 Set level of channel on a running rig

level 43 255               ; check channel is responding to DMX control

# 7. Serial Output Reference

All command responses and errors are printed to the serial monitor. Key patterns:

| Output | Meaning |
| --- | --- |
| NACK 0x0B | Address out of range (NACK reason 11 = FORMAT_ERROR or DATA_OUT_OF_RANGE) |
| No devices found. | Discovery returned zero results. Check bus wiring and termination. |
| GET MFR_CURVE failed | The dimmer did not respond to a curve query. Check RDM is enabled on the dimmer (dmx on) and that the device supports this manufacturer PID. |
| Invalid device index | dev <n> index is out of range. Run discover to refresh the list. |

# 8. RDM PID Reference

The following RDM parameters are used by this controller:

| PID | Name | Usage |
| --- | --- | --- |
| 0x0060 | DMX_START_ADDRESS | addr / addr? |
| 0x0081 | IDENTIFY_DEVICE | identify on/off / identify? |
| 0x00E0 | DMX_PERSONALITY | personality <n> / personality? |
| 0x00E1 | DMX_PERSONALITY_DESCRIPTION | personality? (per-personality descriptions) |
| 0x0090 | DEVICE_INFO | info |
| 0x00C3 | SUPPORTED_PARAMETERS | info (discover) |
| 0x00C4 | PARAMETER_DESCRIPTION | info (discover, mfr PIDs) |
| 0x8000 | MFR_CURVE (manufacturer) | curve / curve? |

Note: PID 0x8000 (MFR_CURVE) is a manufacturer-specific parameter defined by ROLITECH for the S3HW dimmer. Its 6-byte payload carries one curve ID per channel. A value of 0xFF in any byte means “no change” on a SET command.

# 9. Dependencies

The controller sketch requires the following Arduino libraries:

| Library | Notes |
| --- | --- |
| esp_dmx (v4.1.0) | DMX/RDM driver by Mitch Weisbrod. Requires the uart.c patch (rename uart.c to uart.c.bak in src/dmx/hal/). |
| custom_controller.h/.c | ROLITECH helper for rdm_send_get/set_current_curve and RDM_PID_MFR_CURVE. Must be present in the sketch folder. |

Document version: 1.0   —   ROLITECH